Phantom.doc

09/23/1999

GNU Phantom.Security

Copyright (C) 1999 by Joe Thielen <jthielen@accesstoledo.com>

-------------------------------------------

===============================

TABLE OF CONTENTS:

1.  Introduction/Philosophy

2.  GNU Phantom.Security Installation

3.  GNU Phantom.Security Basics

4.  Phantom.conf

5.  Hardware/Electronics Info

6.  Sample Setups & Possibilities

7.  Thanks/Legal Stuff

8.  Comments/Questions/Complaints/Contacting me...

===============================

1.  Introduction/Philosophy

I started the Phantom.Security project as a spin-off of a weak attempt to make

a Linux controlled robot.  But I did learn how to read to and write from the

parallel port, and interface it with the outside using basic electronics.  So

I eventually wired up the company I work at (live at?) and gave it a pretty

neat alarm system.  I looked for other software like this on the Internet

and didn't come up with a whole lot... and nothing in the Open Source category.

So... this is my contribution to the community that has enabled me (and my

company) to do what it does with a next to nothing IT budget!

In accordance with my beliefs, I submitted Phantom.Security to the GNU and now

it's become the product you now have:  GNU Phantom.Security.

2.  GNU Phantom.Security Installation

SOFTWARE:

Aside from compiling the source code (one file) and editing the configuration

file (Phantom.conf), there is little to install.

To compile:

configure

make

make install

This creates the binary needed to run the program.  Editing the Phantom.conf

configuration is detailed in section 4.  Aside from this, there is little to

do software wise.

'make install' defaults to copying the necessary files to /home/Phantom/security

this can be changed by editing the top-level Makefile.am & running aclocal,

autoconf, & automake.  Then run configure, make, & 'make install'.

HARDWARE:

I've included the necessary circuit diagram used to build the interface

between the devices (i.e. motion sensors, door magnets, etc...) and the

computer (or, rather, it's parallel port).  It's a basic simple circuit,

and anyone with some electronics circuit assembly experience should be

able to whip it up.  The circuit can, of course, be modified if your

prerogative defines it :).

3. GNU Phantom.Security Basics

Phantom.Security uses cheap off-the-shelf security devices which can be slightly

modified to work with it.  Radio Shack carries a lot of the this stuff, like

the door magnets and motion sensors.  In most cases it's as easy as taking

the device apart (like the motion sensor) and simply running two wires to the

LED (the little red light).  In others (like some smoke detectors), you may

need step-down transformers and/or rectifiers to convert AC down to a usable

5V DC level.   In any case, if you have some basic electronics knowledge, then

the limits for this package are none!  Door magnets are EXTREMELY easy, and

you can use both Normally Closed and Normally Open devices, the software

is configurable for both via the Phantom.conf file.  The software is run by

simply running the Phantom.Security program.  It first reads the config file,

and then gives a status display which is updated constantly.  It can be run

in the foreground OR background.  To stop it, edit the Phantom.EXIT file and

put only a single 'x' in there.  Save the file.  The program will stop almost

immediately.  Obviously, this may have to be done from another session if

you're running it in the foreground.  You can use the interrupt key to stop it

(i.e. the delete or CTRL-D keys) but it won't write to the .Phantom.Log file

correctly (i.e. you'll get a zero length file), so be careful!  NOTE:  The

Phantom.EXIT file must be in the same directory that the Phantom.Security

program was run from.

Between the time when you start the program, and the time it ends, it is

constantly (depending on machine speed) reading the status of the parallel

port.  If a Smoke Detector device is tripped, then the 'firealert' program

is run.  You should edit this script to do what you want it to do.  Out-of-the-

box it defaults to mailing root.  However, if the computer is in the room with

the fire, that doesn't help much, does it???  So if you have a portable device

which receives e-mail (i.e. a cell phone or pager) put it's e-mail address

in there.  Or you can modify the circuit and software to set off bells and

whistles and stuff!  Whatever you have at your disposal.  This system can be

as small or as large as you want it to be!  It can be run off of slow 486's

and work great!  So setting a machine aside somewhere and letting it run 24/7

for your fire alarm system is very cost effective.  Or you can use an existing

PC or workstation!  You have the possibility of 5 (or 10, with 2 parallel ports)

zones per machine.  In a future version I'll put scalable features in to make

this really easy.

NOTE: To make Phantom.Security work on the SECOND parallel (LPT) port, change

the 379 to 279 in the '#define BASEPORT 0x379' line to '#define BASEPORT

0x279', and re-compile.  The included binary is compiled for the FIRST

parallel port.

4.  Phantom.conf

The format of the Phantom.conf file is as follows:

ZONE#:Enabled/Disabled:Type of Device:NC/NO:Wait State:Location Of Device

-ZONE# is the Zone Number.  '#' is an integer between 1 and 5.

Example: ZONE1

-Enabled/Disabled is the status of the device.  Either Enabled or Disabled.

Example: Enabled

-Type of Device.  So far, Phantom.Security supports the following types of

devices: Door Magnet, Motion Sensor, and Smoke Detector.

Example: Door Magnet

-NC/NO indicated whether the device is Normally Closed or Normally open.

Example: Normally Open

-Wait State is how long to wait after a device has been tripped before it

can be tripped again (the device will still trip, but it will not show up

in the log).

Example: 0

-Location of Device can be used as a general description field.  It makes

the log A LOT easier to read with descriptions.

Example: On main office door

Comments are not allowed in the file.  The format must be followed exactly

or else flaky things might happen :).  Now, you are not limited to just

Door Magnets, Motion Sensors, and Smoke Detectors.  Say if you got this

laser tracking, heat sensing, breath smelling, super-duper apparatus, and it

has 5V Normally Open or Normally Closed outputs, and you want to use it, you

can!  Just say it's a Motion Sensor.  Like wise, if you've got freakin' laser

trip wires, those could function just like a Door Magnet.  Same principal(?).

5.  Hardware/Electronics Info

For the purpose of this discussion, let's assume there are only two types of

devices: passive and active.  A passive device is something like a Door Magnet.

You pass have to provide the signal which passes through it (low OR high).  An

active device is something like a smoke detector or motion sensor.  They both

provide signals (through either a signal jacked from a LED or buzzer...).  Now,

the provided Phantom.Controller diagram is mainly for use with passive devices.

The outer shield provides a +5V DC signal which is passed through the device

(i.e. a door magnet) and when the device is closed, the +5V returns through the

inner pin, which brings the proper pin on the parallel port high.  However,

this won't work with an active device.  But don't fret!  An active device only

requires on simple modification.  In stead of making the outer shield +5V, you

can hook it into ground instead.  Then hook the matching ground from the active

device to it.  Finally, pump the output from the active device (which ideally

should be between +1V and +5V) into the circuit through the inner pin!  And

if you look at the diagram, you can mix and match, so you are not restricted

to 5 passive OR 5 active devices.  Enjoy!

The Phantom.Controller itself will cost between $15-$25 if you shop at Radio

Shack (however, if you're an electronics enthusiast, then you probably know

a cheaper place to get parts).  Door magnets are only $3-$5 @ (again, at Radio

Shack).  Smoke detectors run between $10-$25 @.  Motion sensors are all over

the map.  However the easiest one is also from Radio Shack, the wireless one

they sell for their X10 kit (However, I don't use it as a wireless).  But it's

like $60, so it's pretty expensive.  I've also tried to use the $25 Hawkeye II

from X10 (which I got 2 for 1!), but I haven't been able to get a signal from

the LED to bring the pin high, even using transistors.  I'm having a friend

look at that one...

In any case, I also have a note on smoke detectors.  If you want more than one,

but only have one open space on your Phantom.Controller, then you can buy the

smoke detectors which interconnect to each other, and just run one set of

wires to your Phantom system.  In some of these systems you can have up to

10 detectors or more!  So you can cover a decent area.  The only disadvantage

to doing this is when you're notified, you don't know exactly where the fire

is.

6.  Sample Setups & Possibilities

For a small office area (maybe even a medium size), 5 or 10 input devices is

PLENTY.  The easiest setup would be to put door magnets on all main entrances

and exits.  Then put in one or two motion sensors (depending on how your office

is set up).  Finally, put a smoke detector in for good measure.  You've now

got a full computerized security & fire alarm system!

For those of you wondering "where am I supposed to run these wires???" the only

advice I have is to make use of that wonderful suspended ceiling.  "Well what

if I don't have a suspended ceiling?"  Hmm...  Be creative!  Corners are great

for wires... :).

Once your system is set up, you have to figure out how to make it useful.  Out-

of-the-box it only mails root.  Obviously that's not very useful, but it's the

only thing that all (Unix...) boxes have in common.  However, if your LAN or

WAN is connected to the Internet, and you have a pager or cell phone that

receives e-mail, then you can make good use of this!  You can be notified

within seconds if your place is burning down or being broken into.  And it

works as a silent alarm, so you may have a chance of catching someone in the

act (I do not suggest that you take this duty upon yourself, make sure you call

the proper authorities.  I am not responsible for any vigilante created by the

use of my software... :) )!  The silent alarm part is, of course (cough, cough),

an intended feature, it is NOT a bug.

One neat thing that I will experiment with in the future is an electronic lock.

We have one of these locking an office down the hall, and it works off a remote.

If a wire could be run to it in the right place or the remote modified (using

a transistor mounted where the remote button is supposed to be) they it could

be integrated into the system!  For the time being, I don't have any output

triggers in my software, but if you put it easily in the intruderalert script!

Of course, if you're talented enough to do that, they by all means, go ahead

and modify the source and get to it!  Just do the community a favor and let

everyone know how you did it!  Or send it to me, and I'll post it...

One final note on the program itself.  It has to be somehow started and stopped.

Now, if you start the program and then leave the office, if the door you leave

by has a door magnet, or the lobby has a motion sensor, then you're out of

luck, because you'll get picked up immediately.  So it'd probably make sense to

run the program with a sleep routine before it (in Unix: 'sleep 360').  Since

I run the computers here and created the timecard system, I've got it hooked

up so that after all staff have clocked off a 5 minute timer routine starts.

If no one logs in during that 5 minute period, then the alarm is set.  Now

I've also got my intruderalert program modified so that it waits 3 minutes

before sending out e-mail to see if anyone logs in!  The system runs pretty

much unattended.  Of course, you could do nearly anything, including putting

a numeric keypad at the entrances, if you are so inclined!  Or you could just

walk in an turn the program off.  You'd still get e-mail... but who cares?

You're already there... :)  just make sure your boss doesn't get the e-mail!

7.  Thanks/Legal Stuff

I'd like to thank Richard Stallman <rms@gnu.org>, Brian Youmans <3diff@gnu.org>,

and all the people I've talked to over at the GNU for welcoming me in and making

Phantom.Security a GNU program!

I'd like to thank Riku Saikkonen <Riku.Saikkonen@hut.fi> for his mini-HOWTO on

IO-Port-Programming.  I used his example code almost verbatim in my early

Beta versions.  This is the only document I could find that smacked me in the

face and said "Look idiot, it's that easy!".  And it worked.  Thanks!

This software comes "AS-IS" with NO WARRANTIES expressed or implied.  This

software is free for all to use, copy, edit, compile, distribute, and hack.

I only ask that credit be given where credit is due, and that if a modified

version of this software is distributed, that a copy of the original package

be included.  However, it is free software and open sourced so mod mod mod!

However I am not responsible for any damage the software or circuit diagram

may cause.  Nor am I responsible if the circuit is built wrong and shoots

flames into the persons computer.  Nor am I responsible if the assembler is

drinking Jolt Cola next to a 120V AC transformer and spills it on himself and

the transformer.  However, if the circuit is running off a 5V clean (encased)

power supply, then there shouldn't be any problems.  However if it starts

a nuclear chain reaction in your basement, I cannot be held responsible!

Or if you use it to lock your boss out of the office during a big meeting,

I am not responsible.  Please be safe!

This software is copyrighted, and I've placed it under the GNU Public License.

Please support the GNU and the free software community at http://www.gnu.org!!!

See the file COPYING for details...  here is the distribution statement:

This program is free software; you can redistribute it and/or modify it

under the terms of the GNU General Public License as published by the

Free Software Foundation; either version 2 of the License, or (at your

option) any later version.

8.  BUGS/Comments/Questions/Complaints/Contacting me...

I'd love to hear anything you have to say about the software, so please drop

me an e-mail at jthielen@accesstoledo.com or visit my website at

http://www.joethielen.com

================================================================================